# An arrow rest and its duties

When it comes to target archery the most common rest used are blade rests

Types of rest

- Drop away

- Blade

Use a drop away rest only when shooting different diameters of arrows out of the same bow.

Use a Blade rest when your diameter remains the same .

How to choose the right blade.

- arrow weight

- arrow diameter

Arrow weight : Blade thickness

less than 350 grains - 0.20mm

350-425 grains - 0.25mm

425 and above - 0.30mm

Arrow diameter : Blade size

Most target archers use small diameter arrows so use a 4 mm wide blade for arrows with outer diameter of 4.2 mm till 5 mm.

5 mm wide blade for arrows with outer diameter of 5.3mm till 6 mm.

6 mm for blades of above 6.2 diameter arrows

How to setup the arrow rest

- Take a T scale, the bottom of the scale represents the bottom of the arrow.

- Attach the T scale to the centre serving, place it exactly where the bottom of the T scale is at the same place as the bottom of Berger button hole.

- Connect the Arrow rest to the bow. Make sure that the top of the blade is touching the bottom of the T scale.

- That is your vertical setting on the arrow rest. “Elevation”

- Each bow has its own centre on the horizontal axis. “Windage” or “Center shot”.

- Windage changes according to the arrow and the poundage shot into it.

- To find the right Center shot, hang a piece of string or a tape on the target. try and split the string into two from 10 meters. Adjust the arrow rest Windage until the string is split.