# Thumb Releasers and their functioning

- Make it simple !

- Adjustments

## Make it simple !

We here at imaxx sports wanted a challenge. Being an Indian Company the challenge was not to reinvent the wheel but make it simple.

IMX TR is as simple as a Thumb Releaser gets.

It does its job right and it will do it for a very long time to come.

## Adjustments

Trigger Travel - why is it there.

“Travel” = “Time”

Gives you the ability to control your release process

Shot execution process/ Release process

When your shot is a surprise.

How ?

Either through the process of rotation of the releaser or by relaxing your fingers or by adding extra push and pull in the shot.

When trying to do that you will realise the thumb is automatically applying pressure on the trigger knob.

In any method, the extension of the thumb and applying pressure on to the trigger knob with the use of those techniques, is what is causing the surprise in the shot.

Trigger travel screw next to the trigger knob adjusts Trigger travel. Lock in your trigger and tighten the screw. Once the trigger releases automatically, take 0.5 turns out, that is your “Zero Travel”.

Zero Travel helps in keeping the latency between shot execution process and the releaser minimum. Thereby each shot reacting to your sensitivity setting quickly.

Having travel gives you the time to control the process better.

It is the same when it comes to sensitivity adjustments too. When tight(turning in) the effort needed to the set off the releaser is more.

- Start off at just 2 or 3 meters from the target bale.

- Set the Trigger Travel - 1 turn out.

- Set Sensitivity screw all the way in, Full tight.

- Close your eyes and feel the shot.

- Take 1 full turn out.

- Close your eyes and take a shot.

- Similarly keep playing with the sensitivity screw until you find a setting where the effort needed is very consistent shot after shot.

- Go to 10 meters, try shooting with the same settings. You will definitely find that the releaser has gotten more difficult to execute.

- This is where you play with the trigger travel. Put the travel at Zero to execute it faster .

Take your time to get an understanding of the where theSight Pin is before the shot goes off.

Whether the aim has settled and it is just the shot execution that needs to happen or whether the shot execution happens immediately the moment you realise the pin settling on the target.

YOU WANT THE FIRST, everything has settled and then shot execution happens. To settle everything, kindly contact expert coaches who can help you with your technique and equipment settings.